| Date: | 04-06-2020 | Name: | Rohan shetty |
| --- | --- | --- | --- |
| Course: | Digital Design Using HDL | USN: | 4AL17EC079 |
| Topic: | Hardware Modeling using Verilog Implement T Flip-Flop | Semester & Section: | 6th & ‘B’ |
| Github Repository: | rohan-shetty-online-courses |  |  |

FORENOON SESSION DETAILS

Image of session

Report –Hardware Modeling Using Verilog Objective of Hardware Modeling Using Verilog - Learn about the Verilog hardware description language. - Understand the difference between behavioral and structural design styles. - Learn to write test benches and analyze simulation results. - Learn to model combinational and sequential circuits, - Distinguish between good and bad coding practices. - Case studies with some complex designs. VLSI Design Process - Design complexity increasing rapidly - Increased size and complexity - Fabrication technology improving - CAD tools are essential - Conflicting requirements like area, speed, and energy consumption

- The present trend - Standardize the design flow - Emphasis on low-power design, and increased performance Moore’s Law - Exponential growth - Design complexity increases rapidly - Automated tools are essential - Must follow well defined design flow Standardized design procedure - Starting from the design idea down to the actual implementation. Encompasses many steps: - Specification Synthesis - Simulation - Layout Testability analysis - and many more Need to use Computer Aided Design (CAD) tools. - Hardware Description Language (HDL) - Based on HDL provide formats for representing the outputs of various design steps - A CAD tool transforms its HDL input into a HDL output that contains more detailed information about the hardware. - Behavioral level to register transfer level - Register transfer level to gate level - Gate level to transistor level - Transistor to the layout level Two Competing HDL’s - Verilog - VHDL Behavioral design - Specify the functionality of the design in terms of its behavior. - Various ways of specifying: - Boolean expression or truth table. - Finite-state machine behavior (e.g. state transition diagram or table). - In the form of a high-level algorithm. - Needs to be synthesized into more detailed specifications for hardware realization, Data path design - Generate a netlist of register transfer level components, like registers, adders, multipliers, multiplexers, decoders, etc. - A netlist is a directed graph, where the vertices indicate components, and the edges indicate interconnections.

app.py

from flask import Flask, render_template, request from flask.ext.sqlalchemy import SQLAlchemy

from send_email import send_email from sqlalchemy.sql import func

app=Flask( 	name 	) app.config['SQLALCHEMY_DATABASE_URI']='postgresql://postgres:postgr es123@localhost/height_collector'

db=SQLAlchemy(app)

class Data(db.Model):

tablename 	="data" id=db.Column(db.Integer, primary_key=True)

email_=db.Column(db.String(120), unique=True) height_=db.Column(db.Integer)

def 		init 	(self, email_, height_): self.email_=email_ self.height_=height_

@app.route("/") def index():

return render_template("index.html")

@app.route("/success", methods=['POST']) def success():

if request.method=='POST': email=request.form["email_name"] height=request.form["height_name"] print(email, height)

if db.session.query(Data).filter(Data.email_ == email).count()== 0:

data=Data(email,height) db.session.add(data) db.session.commit()

average_height=db.session.query(func.avg(Data.height_)).scalar() average_height=round(average_height, 1)

count = db.session.query(Data.height_).count() send_email(email, height, average_height, count) print(average_height)

return render_template("success.html")

return render_template('index.html', text="Seems like we got something from that email once!")